PROJECT DOCUMENTATION: DIY XINPUT STEERING WHEEL

This project uses the ArduinoXInput library to allow specific microcontrollers to emulate an official Xbox 360 controller. By appearing as a native XInput device, the steering wheel works automatically with Windows and racing simulators without needing extra mapping software.

Hardware Compatibility List

A board must have Native USB support to work with this code. This means the main chip handles the USB connection directly.

🟢Compatible Boards (Native USB)

These boards are fully supported and will work for your steering wheel and pedals:

- Arduino Pro Micro: Highly recommended for compact builds. It uses the ATmega32u4 chip and fits easily inside a steering wheel hub.

- Arduino Leonardo: The standard choice for DIY wheels. It offers many analog pins for high-precision steering and pedal inputs.

- Arduino Micro: A mid-sized version of the Leonardo with full support.

- Teensy 4.0 and 4.1: Extremely fast boards that offer the lowest possible input lag for competitive racing.

- Teensy LC, 3.2, 3.5, and 3.6: Reliable high-performance alternatives.

- Arduino Zero: Uses a modern ARM chip with native USB capabilities.

- Arduino Uno R4 Minima: The newest version of the Uno that includes native USB support.

- SparkFun Pro Micro: A common and reliable variant of the Pro Micro architecture.

- Arduino Esplora: Built-in game-style inputs using the Leonardo chip.

❌ Incompatible Boards (No Native USB)

These boards use a separate bridge chip to talk to the PC. They cannot be seen as an Xbox controller and will not work for this project:

- Arduino Uno (R3 and older)

- Arduino Nano

- Arduino Mega 2560

- Arduino Pro Mini